Exc 2.

Topologia

6 / 7

PC1- 10.1.1.2 Vlan1

PC2- 10.1.2.2	Vlan2

PC3- 10.1.2.3 Vlan2

PC4- 10.1.2.2 Vlan2

ESW1- Interfaces Vlan

Inicialmente é usado estes IPs como Gateways para respectivas Vlans

Vlan1 - 10.1.1.1

Vlan2 - 10.1.2.1

Vlan3- 10.1.3.1

Interface f0/0 - 10.2.1.1

ESW2- Interfaces Vlan

Vlan1- 10.1.1.254

Vlan2- 10.1.2.254

Vlan3- 10.1.3.254

Interface f0/0 - 10.2.1.2

R1- Interface f0/0 - DHCP

Interface f0/1 - 10.2.1.2

Confirmaçáo de Ligaçao entre Vlans

root@ubun-1:~# ping 10.1.2.2

PING 10.1.2.2 (10.1.2.2) 56(84) bytes of data.

64 bytes from 10.1.2.2: icmp_seq=1 ttl=63 time=21.0 ms

64 bytes from 10.1.2.2: icmp_seq=1 ttl=63 time=30.0 ms (DUP!)

64 bytes from 10.1.2.2: icmp_seq=2 ttl=63 time=22.0 ms

64 bytes from 10.1.2.2: icmp_seq=3 ttl=63 time=13.0 ms

--- 10.1.2.2 ping statistics ---

3 packets transmitted, 3 received, +1 duplicates, 0% packet loss, time 2007ms

rtt min/avg/max/mdev = 13.001/21.500/30.001/6.020 ms

root@ubun-1:~# ping 10.1.3.2

PING 10.1.3.2 (10.1.3.2) 56(84) bytes of data.

64 bytes from 10.1.3.2: icmp_seq=1 ttl=63 time=59.1 ms

64 bytes from 10.1.3.2: icmp_seq=1 ttl=63 time=72.0 ms (DUP!)

64 bytes from 10.1.3.2: icmp_seq=2 ttl=63 time=22.1 ms

^C

--- 10.1.3.2 ping statistics ---

2 packets transmitted, 2 received, +1 duplicates, 0% packet loss, time 1000ms

rtt min/avg/max/mdev = 22.079/51.046/72.001/21.153 ms

Confirmaçáo de Ligaçao entre Redes

Aqui foi usado RIP como Protocolo de Encaminhamento

root@ubun-1:~# ping 10.2.1.254

PING 10.2.1.254 (10.2.1.254) 56(84) bytes of data.

64 bytes from 10.2.1.254: icmp_seq=1 ttl=254 time=69.9 ms

64 bytes from 10.2.1.254: icmp_seq=2 ttl=254 time=22.6 ms

64 bytes from 10.2.1.254: icmp_seq=3 ttl=254 time=19.1 ms

64 bytes from 10.2.1.254: icmp_seq=4 ttl=254 time=57.6 ms

^C

--- 10.2.1.254 ping statistics ---

4 packets transmitted, 4 received, 0% packet loss, time 3005ms

rtt min/avg/max/mdev = 19.085/42.311/69.941/21.926 ms

8 / 9

Configuração NAT no R1

R1#show run | section ip nat

ip nat outside

ip nat inside

ip nat inside source list 1 interface FastEthernet0/0 overload

R1#show run | section interface

interface FastEthernet0/0

ip address dhcp

ip nat outside

ip virtual-reassembly

duplex auto

speed auto

interface FastEthernet0/1

ip address 10.2.1.254 255.255.255.0

ip nat inside

ip virtual-reassembly

duplex auto

speed auto

ip nat inside source list 1 interface FastEthernet0/0 overload

R1#show access-lists

Standard IP access list 1

10 permit 10.1.1.0, wildcard bits 0.0.0.255 (3 matches)

20 permit 10.1.2.0, wildcard bits 0.0.0.255

30 permit 10.1.3.0, wildcard bits 0.0.0.255

40 permit 10.2.1.0, wildcard bits 0.0.0.255

R1#sh ip route

Gateway of last resort configurado por dhcp

Gateway of last resort is 192.168.122.1 to network 0.0.0.0

C    192.168.122.0/24 is directly connected, FastEthernet0/0

10.0.0.0/24 is subnetted, 4 subnets

R       10.1.3.0 [120/1] via 10.2.1.2, 00:00:14, FastEthernet0/1

[120/1] via 10.2.1.1, 00:00:21, FastEthernet0/1

C       10.2.1.0 is directly connected, FastEthernet0/1

R       10.1.2.0 [120/1] via 10.2.1.2, 00:00:14, FastEthernet0/1

[120/1] via 10.2.1.1, 00:00:21, FastEthernet0/1

R       10.1.1.0 [120/1] via 10.2.1.2, 00:00:14, FastEthernet0/1

[120/1] via 10.2.1.1, 00:00:23, FastEthernet0/1

S*   0.0.0.0/0 [1/0] via 192.168.122.1

Confirmação de acesso ao exterior

root@ubun-1:~# ping 8.8.8.8

PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data.

64 bytes from 8.8.8.8: icmp_seq=1 ttl=252 time=47.0 ms

64 bytes from 8.8.8.8: icmp_seq=2 ttl=252 time=49.6 ms

64 bytes from 8.8.8.8: icmp_seq=3 ttl=252 time=62.1 ms

64 bytes from 8.8.8.8: icmp_seq=4 ttl=252 time=62.0 ms

^C

--- 8.8.8.8 ping statistics ---

4 packets transmitted, 4 received, 0% packet loss, time 3009ms

rtt min/avg/max/mdev = 47.000/55.178/62.107/6.936 ms

R1#sh ip nat translation

Pro Inside global      Inside local       Outside local      Outside global

icmp 192.168.122.3:46  10.1.1.2:46        8.8.8.8:46         8.8.8.8:46

10.

Ao simular uma falha no EWS2 perdemos redundancia mas mantemos conecção pois o gateway defenido esta no .1 das vlans

No entanto ao simular a falha no ESW1:

root@ubun-1:~# ping 10.1.2.2

PING 10.1.2.2 (10.1.2.2) 56(84) bytes of data.

^C

--- 10.1.2.2 ping statistics ---

3 packets transmitted, 0 received, 100% packet loss, time 2079ms

Como não temos HSRP configurado a network fica dependente de um só router para conecções fora da vlan

12.

ESW1#show standby brief

P indicates configured to preempt.

|

Interface   Grp Prio P State    Active          Standby         Virtual IP

Vl1         1   120  P Active   local           10.1.1.254      10.1.1.10

Vl2         1   120  P Active   local           10.1.2.254      10.1.2.10

Vl3         1   120  P Active   local           10.1.3.254      10.1.3.10

ESW2#show standby brief

P indicates configured to preempt.

|

Interface   Grp Prio P State    Active          Standby         Virtual IP

Vl1         1   100  P Standby  10.1.1.1        local           10.1.1.10

Vl2         1   100  P Standby  10.1.2.1        local           10.1.2.10

Vl3         1   100  P Standby  10.1.3.1        local           10.1.3.10

root@ubun-1:~# ip r

default via 10.1.1.10 dev eth0

10.1.1.0/24 dev eth0 proto kernel scope link src 10.1.1.2

root@ubun-1:~# ping 10.1.1.10

PING 10.1.1.10 (10.1.1.10) 56(84) bytes of data.

64 bytes from 10.1.1.10: icmp_seq=1 ttl=255 time=84.0 ms

64 bytes from 10.1.1.10: icmp_seq=2 ttl=255 time=16.5 ms

64 bytes from 10.1.1.10: icmp_seq=3 ttl=255 time=24.8 ms

^C64 bytes from 10.1.1.10: icmp_seq=4 ttl=255 time=6.72 ms

--- 10.1.1.10 ping statistics ---

4 packets transmitted, 4 received, 0% packet loss, time 3001ms

rtt min/avg/max/mdev = 6.723/33.021/84.003/30.123 ms

root@ubun-1:~# ping 10.2.1.254

PING 10.2.1.254 (10.2.1.254) 56(84) bytes of data.

64 bytes from 10.2.1.254: icmp_seq=2 ttl=254 time=28.0 ms

64 bytes from 10.2.1.254: icmp_seq=3 ttl=254 time=26.5 ms

64 bytes from 10.2.1.254: icmp_seq=4 ttl=254 time=32.7 ms

64 bytes from 10.2.1.254: icmp_seq=5 ttl=254 time=44.0 ms

^C

--- 10.2.1.254 ping statistics ---

5 packets transmitted, 4 received, 20% packet loss, time 4048ms

rtt min/avg/max/mdev = 26.519/32.805/44.001/6.854 ms

root@ubun-1:~# ping 8.8.8.8

PING 8.8.8.8 (8.8.8.8) 56(84) bytes of data.

64 bytes from 8.8.8.8: icmp_seq=1 ttl=252 time=57.2 ms

64 bytes from 8.8.8.8: icmp_seq=2 ttl=252 time=49.5 ms

64 bytes from 8.8.8.8: icmp_seq=3 ttl=252 time=66.0 ms

64 bytes from 8.8.8.8: icmp_seq=4 ttl=252 time=46.0 ms

^C

--- 8.8.8.8 ping statistics ---

4 packets transmitted, 4 received, 0% packet loss, time 3007ms

rtt min/avg/max/mdev = 46.002/54.671/66.003/7.685 ms

R: Agora com os gateway dos PCs actualizados e HSRP configurado existe redundancia tambem nos routers e se um falhar o outro